WEST YANGON TECHNOLOGICAL UNIVERSITY

DEPARTMENT OF COMPUTER ENGINEERING AND INFORMATION TECHNOLOGY

DESIGN AND IMPLEMENTATION OF A SECURE LINUX SERVER FOR THE IT DEPARTMENT USING ROCKY LINUX 10

BY

GROUP-III

FOURTH YEAR PROJECT

October, 2026

YANGON

TABLE OF CONTENTS

Page

ABSTRACT										  i

TABLE OF CONTENTS								  ii LIST OF FIGURES									  iii

LIST OF TABLE									  iv LIST OF ABBREVIATIONS								   v

| CHAPTER TITLE |  |  |
| --- | --- | --- |
| - INTRODUCTION | 1 |  |
| 1.1.1 Background | 1 |  |
| 1.1.2 Problem Statement | 1 |  |
| 1.1.3 Aimof the Project | 1 |  |
| 1.1.4 Objectives | 2 |  |
| 1.1.5 Significance of the Project | 2 |  |
| 1.2.1 Linux | 3 |  |
| 1.2.2 Linux System Adminstration | 3 |  |
| 1.2.3 Rocky Linux | 3 |  |
| 1.2.4 Client-Sever Architecture | 3 |  |
| 2 SYSTEM AND IMPLEMENTATION | 4 |  |
| 2.1 System Overview | 4 |  |
| 2.1.2 Server Installation | 5 |  |
| 2.1.3.Hostname Configuration | 5 |  |
| 2.1.4 Network Configuration | 5 |  |
| 3. 3.1.1 SSH | 6 |  |
| 3.1.2 Nginx | 6 |  |
| 3.1.3 MariaDB | 7 |  |
| 3.2.1 User and Group Management | 8 |  |
| 3.2.2 Firewalld | 8 |  |
| 3.2.3 SELinux | 8 |  |
| - BACKUP AND MAINTENANCE | 9 |  |
| 4.1 Backup | 9 |  |
| 4.1.1 System Monitoring | 9 |  |
| - TESTING AND RESULT | 10 |  |
| 5.1.1 Connectivity Test | 10 |  |
| 5.1.2 SSH Test | 10 |  |
| 5.1.3 Nginx Test | 10 |  |
| 5.1.4 Samba Test | 10 |  |
| 5.1.5 MariaDB Test | 10 |  |
| 5.1.6 Security Test | 10 |  |
| 5.1.7 Backup & Restore Test | 10 |  |
| 5.2 Discussion | 11 |  |
| 5.3 Conclusion And Future Work | 12 |  |
| 5.3.1 Conclusion | 12 |  |
| 5.3.2 Future Work | 12 |  |

ABSTRACT

This project focuses on the implementation of a centralized Linux server using Rocky Linux 10 in a school computer laboratory environment. The system consists of one Rocky Linux server and four client computers connected through a local network. The server is configured to provide centralized services including web hosting, file sharing, database management, and remote administration.

Nginx is implemented as the web server, Samba is used for file sharing, MariaDB is used for database management, and OpenSSH provides secure remote administration. User and group management, Firewalld, SELinux, and access permissions are also configured to improve system security. An automated backup mechanism is included to protect important server data.

The implemented system is tested using client-server connectivity, remote login, web access, file sharing, database access, security, and backup tests. The project demonstrates the practical application of Linux System Administration concepts and provides a foundation that can be extended for centralized services in a school or departmental laboratory.

TABLE OF CONTENTS

LIST OF FIGURES

Figure 4.1 	Overall centralized server architecture

Figure 4.2 	Logical layer relationship

Figure 4.3	 Proposed internal network architecture

Figure A.1 	Evidence placement guide

LIST OF TABLE

Table 1.1 	Relationship between problems and objectives

Table 3.1 	Functional requirements

Table 3.2 	Nonfunctional requirements

Table 3.3 	Proposed hardware requirements

Table 4.1 	Role and access design

Table 6.1 	Security control matrix

Table 6.2 	Security testing record

Table 7.1 	System testing record

Table 8.1 	Objective evaluation record

Table 9.1	 Limitations and effects

LIST OF ABBREVIATIONS

| Abbreviation | Meaning |
| --- | --- |
| CEIT | Computer Engineering and Information Technology |
| CLI | Command Line Interface |
| DAC | Discretionary Access Control |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | Hypertext Transfer Protocol Secure |
| IP | Internet Protocol |
| MAC | Mandatory Access Control |
| RBAC | Role Based Access Control |
| RDBMS | Relational Database Management System |
| SELinux | Security Enhanced Linux |
| SMB | Server Message Block |
| SSH | Secure Shell |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

CHAPTER 1

INTRODUCTION

1.1.1 Background

In modern educational environments, computer laboratories require reliable methods for managing users, files, applications, and network services. Managing these resources separately on individual computers can make administration difficult and inefficient.

A centralized server can provide common services and resources to multiple client computers through a network. Linux provides a flexible and cost-effective platform for implementing server systems. Rocky Linux is a suitable enterprise-oriented Linux distribution for server administration and can be used to provide various network services.

Therefore, this project focuses on implementing a centralized Linux server using Rocky Linux 10 in a school laboratory environment.

1.1.2 Problem Statement

In a computer laboratory, files, services, and user access may need to be managed across multiple computers. Without centralized management, administrators may need to configure and maintain each computer separately.
	This project addresses this issue by implementing a centralized Rocky Linux server that provides common services to multiple client computers through a local network.

1.1.3 Aim of the Project

The main aim of this project is to design and implement a centralized Linux server using Rocky Linux 10 for providing and managing network services in a school laboratory environment.

1.1.4 Objectives

To install and configure Rocky Linux 10 as a server operating system.

To establish communication between the server and client computers.

To implement centralized web, file-sharing, and database services.

To configure SSH for remote server administration.

To manage users, groups, and access permissions.

To configure FIREWALLD and SELINUX for server security.

To implement automated data backup.

To test and evaluate the implemented system.

1.1.5 Significance of the Project

This project demonstrates the practical application of Linux System Administration concepts in a real laboratory environment. The implemented system provides a centralized platform for managing network services and users.

The project can also serve as a foundation for future development of centralized IT services within a school or department. The system can be further extended with additional services such as DNS, DHCP, centralized monitoring, and high-availability solutions.

1.2.1 Linux

Linux is an open-source operating system widely used for servers, personal computers, networking devices, and other computing environments. Linux provides powerful command-line tools and system administration features for managing users, processes, networks, storage, and services.

1.2.2 Linux System Administration

Linux System Administration is the process of installing, configuring, maintaining, monitoring, and securing Linux-based computer systems. It includes user management, network configuration, service management, storage management, security, backup, and troubleshooting.

1.2.3 Rocky Linux

Rocky Linux is an open-source enterprise Linux distribution designed to provide a stable and reliable operating system for server environments. In this project, Rocky Linux 10 is used as the server operating system.

1.2.4 Client-Server Architecture

The client-server architecture consists of client devices that request services and a server that provides those services through a network. In this project, four client computers communicate with a centralized Rocky Linux server.

CHAPTER 2

SYSTEM DESIGN AND IMPLEMENTTION

2.1.1  System Overview

School Lab Network
│
┌────────────────┼────────────────┐
│ │ │
Client 1 Client 2 Client 3
│ │ │
└────────────────┼────────────────┘
│
Client 4
│
↓
Rocky Linux 10
Server
│
┌────────────────┼────────────────┐
│ │ │
Nginx Samba MariaDB
│ │ │
Web Service File Sharing Database
│
↓
Security & Backup

Explanation

The proposed system consists of one Rocky Linux 10 server and four client computers. All devices are connected through the same local network. The Rocky Linux server provides centralized services to the client computers. Nginx provides web services, Samba provides file sharing, MariaDB provides database services, and OpenSSH is used for remote administration. Firewalld, SELinux, and user permissions are applied to improve security.

2.1.2 Server Installation

Rocky Linux 10 was installed on the designated server computer. During installation, the required user account, hostname, storage configuration, and network settings were configured.

2.1.3 Hostname Configuration

sudo hostnamectl set-hostname dept-server

hostnamectl

Explanation

The server hostname was changed to dept-server to provide a meaningful and identifiable name within the laboratory environment.

2.1.4 Network Configuration

| Device | Hostname | IP Address |
| --- | --- | --- |
| Server | dept-server | Actual IP |
| Client 1 | client-01 | Actual IP |
| Client 2 | client-02 | Actual IP |
| Client 3 | client-03 | Actual IP |
| Client 4 | client-04 | Actual IP |

CHAPTER 3

SERVER SERVICES AND SECURITY CONFIGURATION

3.1.1 SSH

sudo dnf install openssh-server -y

sudo systemctl enable --now sshd

Firewall:

sudo firewall-cmd --permanent --add-service=ssh

sudo firewall-cmd --reload

ssh username@server-ip

Explanation

OpenSSH was configured to allow authorized users to remotely administer the Rocky Linux server through the network.

3.1.2 Nginx

sudo dnf install nginx -y

sudo systemctl enable --now nginx

http://SERVER-IP

Explanation

Nginx was configured as the web server. Clients can access the hosted web service through the server's IP address.

3.1.3 Samba

sudo dnf install samba samba-client -y

Samba was implemented to provide centralized file sharing between the server and authorized client computers.

3.1.4 MariaaDB

MariaDB was configured as the database management service. A database and authorized database user were created to demonstrate centralized database management.

3.1.5 User and Group Management

sudo useradd -m student1
sudo passwd student1

Group:
sudo groupadd department
sudo usermod -aG department student1

Explanation

User and group management was implemented to control access to server resources based on user roles.

3.2..2 Firewalld

sudo firewall-cmd --state
sudo firewall-cmd --list-all

Explanation

Firewalld was configured to control incoming network connections and allow only the required services.

3.2.3 SELinux

getenforce

Expected:
Enforcing

Explanation

	SELinux was maintained in enforcing mode to provide an additional layer of access control and security for the server.

CHAPTER 4

BACKUP AND MAINTENANCE

4.1.1 Backup



Website files
Samba shared files
Database
Important configuration files

Explanation

A backup mechanism was implemented to protect important server data and support recovery in case of data loss or system failure.

4.1.2 System Monitoring

top
free -h
df -h
ss -tulpn
systemctl --failed

Explanation

System monitoring commands were used to observe CPU, memory, disk usage, network ports, and failed services.

CHAPTER 5

TESTING AND RESULT

5.1 Connectivity Test
ping SERVER-IP

Expected: Successful response

Result: Pass / Fail

5.1.2 SSH Test
ssh username@SERVER-IP

Expected: Authorized user can remotely log in.

5.1.3 Nginx Test

Browser: http://SERVER-IP

Expected: Web page accessible.

5.1.4 Samba Test

Expected: Authorized clients can access shared files.

5.1.5 MariaDB Test

Expected: Authorized database user can connect to the database.

5.1.6 Security Test

Expected: Unauthorized access is denied and only required services are accessible.

5.1.7 Backup & Restore Test

Expected: Backup is successfully created and selected data can be restored.

Test Result Table

| Test Expected Result | Actual Result Status |
| --- | --- |
| Ping Successful | Actual result Pass |
| SSH Login successful | Actual result Pass |
| Nginx Website accessible | Actual result Pass |
| Samba File sharing works | Actual result Pass |
| MariaDB Database accessible | Actual result Pass |
| Firewall Unauthorized access blocked | Actual result Pass |
| Backup Backup/restore successful | Actual result Pass |

5.2 Discussion

The implementation demonstrated that Rocky Linux 10 can be configured as a centralized server for providing multiple network services in a laboratory environment. The server successfully provided web, file-sharing, database, and remote administration services to authorized clients.

The project also demonstrated the importance of user permissions, firewall configuration, SELinux, backup, and system monitoring in maintaining a reliable server environment.

5.3 Conclusion And Future Work

5.3.1 Conclusion

This project successfully demonstrated the implementation of a centralized Linux server using Rocky Linux 10 in a school laboratory environment. One server and four client computers were connected through a local network, and several server services were implemented and tested.

The project provided practical experience in Linux server installation, network configuration, user management, service configuration, security, backup, and troubleshooting. The implemented system provides a foundation that can be further developed for centralized IT services within a school or department.

5.3.2 Future Work

The system can be further extended in future projects by implementing centralized DNS and DHCP services, web-based server management, centralized monitoring, improved backup and recovery, and high-availability clustering.

REFERENCES

Rocky Enterprise Software Foundation, Rocky Linux Documentation.
https://docs.rockylinux.org/
Red Hat Customer Content Services, Red Hat Enterprise Linux 10 Documentation.
https://docs.redhat.com/en/documentation/red_hat_enterprise_linux/10/
NGINX, NGINX Documentation.
https://docs.nginx.com/
MariaDB Foundation, MariaDB Documentation.
https://mariadb.com/docs/
Samba Team, Samba Documentation.
https://www.samba.org/samba/docs/
OpenSSH Project, OpenSSH Documentation.
https://www.openssh.com/

PARTICIPANTS LISTS OF GROUP-III

| NO. | NAME | ROLL NUMBER | SIGN |
| --- | --- | --- | --- |
| 1. | Mg Hlyan Htet Aung | IV.IT-24 (Leader) |  |
| 2. | Ma Mya Kyaw Zin Thant | IV.IT-9 |  |
| 3. | Ma Aye Myat San | IV.IT-12 |  |
| 4. | Ma Zin Yamin Moe Aung | IV.IT-18 |  |
| 5. | Ma Myat Su Mon | IV.IT-22 |  |

WEST YANGON TECHNOLOGICAL UNIVERSITY

DEPARTMENT OF COMPUTER ENGINEERING AND

INFORMATION TECHNOLOGY

We hereby confirm that we have carefully reviewed this project report and recommend it to the University Steering Committee for Undergraduate Studies for approval. The project entitled “Design and Implementation of a Secure Linux Server for the IT Department Using Rocky Linux 10”, submitted by Fourth Year Group-III (October, 2026) to the Department of Computer Engineering and Information Technology.

Board of Examiners:

- Dr. Cho Cho Myint

Professor and Head

Department of Computer Engineering

Information Technology				        ..……………………

West Yangon Technological University		    	     (Chairman)

- Dr. Honey Htun

Associate Professor

Department of Computer Engineering

and Information Technology				         ..…………………..

West Yangon Technological University		   	     (Supervisor)